CASE STUDY: COMPARING THE OSI AND TCP/IP NETWORK MODELS: LAYERS, RESPONSIBILITIES, PROTOCOLS, AND PRACTICAL APPLICATIONS

Abstract

Communication between devices relies on common rules that allow different systems to exchange information. Two widely used ways to explain network communication are the Open Systems Interconnection (OSI) model and the Transmission Control Protocol/Internet Protocol (TCP/IP) model. The OSI model divides communication into seven conceptual layers, whereas the TCP/IP model groups network functions into four layers and reflects the protocol suite used across the Internet. This case study reviews their structures, roles, advantages, limitations, and use in activities such as web access, email, cloud storage, and connected devices. [1][2]

1. Introduction

Computer networks support everyday activities such as messaging, online learning, shopping, and sharing files. To let equipment and software from different vendors communicate, networking standards describe how information should be prepared, transmitted, routed, and received. Two important frameworks used to explain these processes are the OSI Reference Model and the TCP/IP model.

The Open Systems Interconnection (OSI) model, standardized by ISO, divides network communication into seven layers. It is especially useful as a common reference when learning networking concepts, planning systems, or narrowing down faults. [1][3]

TCP/IP developed through work associated with the ARPANET and became the protocol suite used for Internet communication. Its four-layer view groups related tasks together and describes protocols that operate across interconnected networks. [2][4]

Comparing the models helps learners understand where particular networking tasks occur and gives technicians a systematic way to design, explain, and troubleshoot communication.

2. Objectives of the Case Study

The main objectives of this case study are to:

- Describe the OSI and TCP/IP models.

- Compare their layers, responsibilities, and commonly used protocols.

- Identify the practical benefits and limitations of both models.

- Relate the models to familiar network-based services.

- Explain how shared standards support communication between different systems.

3. Brief History of OSI and TCP/IP Models

ISO published the OSI reference model as a framework for organizing network functions. Its seven-layer structure provides a consistent vocabulary for describing what happens as data moves between applications and network media. Although real-world protocol stacks do not always match the model layer for layer, it remains valuable in education and troubleshooting. [1][3]

TCP/IP evolved from networking research connected with ARPANET and was designed around protocols that could be implemented on real systems. Its layered architecture is documented in Internet standards, including RFC 1122, and forms the basis of communication across the Internet. [2][4]

4. Overview of the OSI Model

The OSI (Open Systems Interconnection) model divides communication into seven layers. Each layer focuses on a set of related tasks and provides services to the layer above it. This separation helps explain the journey of data and makes it easier to isolate network problems. [1][3]

| Layer | Main responsibility |
| --- | --- |
| Application | Provides network services to user applications. |
| Presentation | Handles data representation, encryption, and compression. |
| Session | Coordinates application communication sessions. |
| Transport | Supports end-to-end delivery and error handling. |
| Network | Provides logical addressing and routing. |
| Data Link | Frames data and uses local-link addressing. |
| Physical | Transmits bits over the physical medium. |

5. Functions of OSI Layers

1. Application Layer

Provides network-facing services used by applications such as browsers, email programs, and file-transfer tools. Examples include HTTP, FTP, SMTP, and DNS.

2. Presentation Layer

Represents data in a suitable format and may support encoding, compression, and encryption-related functions.

3. Session Layer

Coordinates the start, maintenance, and termination of communication sessions between applications.

4. Transport Layer

Supports communication between endpoints through segmentation, ordering, flow control, and error handling. TCP and UDP are common transport protocols.

5. Network Layer

Uses logical addresses and routing to move packets between networks; IP is central to this function, while ICMP supports diagnostics and error reporting.

6. Data Link Layer

Organizes data into frames for a local link and uses hardware addresses where applicable. Ethernet and Wi-Fi provide common link technologies.

7. Physical Layer

Transmits bits using a physical medium, for example copper cable, fibre-optic cable, or radio signals. [1][3]

6. Overview of the TCP/IP Model

The TCP/IP (Transmission Control Protocol/Internet Protocol) model describes the layered protocol suite used for communication over interconnected networks. It commonly has four layers: Application, Transport, Internet, and Network Access. Several functions that are separate in the OSI model are combined here, reflecting the practical design of Internet protocols. [2][4]

| Layer | Main responsibility |
| --- | --- |
| Application | Offers services used by network applications. |
| Transport | Carries data between application processes. |
| Internet | Addresses and forwards packets between networks. |
| Network Access | Moves frames over the local network medium. |

7. Functions of TCP/IP Layers

1. Application Layer

The Application layer provides network services to programs and combines functions that the OSI model places in its Application, Presentation, and Session layers.

Common protocols: HTTP, HTTPS, FTP, SMTP, DNS, DHCP

2. Transport Layer

This layer carries data between application processes and may provide reliability, ordering, and flow management depending on the protocol.

Protocols:

- TCP — connection-oriented transport with reliable, ordered delivery.

- UDP — connectionless transport with low overhead and no built-in delivery guarantee.

3. Internet Layer

The Internet layer provides logical addressing and forwards packets across multiple interconnected networks.

Protocols:

- IP

- ICMP

- ARP

4. Network Access Layer

This layer connects the Internet protocol stack to the local network technology, handling link framing and access to the transmission medium.

Technologies:

- Ethernet

- Wi-Fi

- PPP

- Fiber Optics

Because it is widely implemented and can operate over different link technologies, TCP/IP is suitable for networks ranging from small local networks to the global Internet. [2][4]

8. Layer-wise Mapping of OSI and TCP/IP Models

The two models describe many of the same networking tasks, but they group them differently.

| OSI Model | TCP/IP Model |
| --- | --- |
| Application | Application |
| Presentation | Application |
| Session | Application |
| Transport | Transport |
| Network | Internet |
| Data Link | Network Access |
| Physical | Network Access |

In the common four-layer mapping, the OSI Application, Presentation, and Session layers correspond to TCP/IP's Application layer. OSI Physical and Data Link functions are grouped into TCP/IP's Network Access layer. This is a useful comparison rather than a strict one-to-one match for every protocol. [1][2]

9. Comparison Between OSI and TCP/IP Models

9.1 Comparison with Respect to Layers

| Parameter | OSI Model | TCP/IP Model |
| --- | --- | --- |
| Number of Layers | 7 | 4 |
| Developed / Standardized By | ISO | DARPA / Internet research community |
| Architecture | Conceptual reference framework | Practical protocol suite |
| Typical Use | Learning and troubleshooting | Internet and network communication |
| Layer Boundaries | More distinctly separated | Grouped around protocol functions |

9.2 Comparison with Respect to Functions

| OSI Model | TCP/IP Model |
| --- | --- |
| Presentation and Session are distinct layers | Their functions are grouped under Application |
| Physical and Data Link are distinct layers | Their functions are grouped under Network Access |
| Emphasizes a reference structure | Emphasizes protocols used in real networks |
| Separates functions for analysis | Combines related tasks where practical |

9.3 Comparison with Respect to Protocols

| Layer | Common protocols / technologies |
| --- | --- |
| Application | HTTP, HTTPS, FTP, SMTP, DNS |
| Transport | TCP, UDP |
| Internet / Network | IP, ICMP |
| Network Access | Ethernet, Wi-Fi, PPP |

10. Advantages and Limitations

Advantages of the OSI Model

- Makes network responsibilities easier to separate and discuss.

- Offers a clear framework for learning and systematic troubleshooting.

- Supports modular thinking when designing or analysing network systems.

- Provides a shared vocabulary for students, engineers, and developers.

Limitations of the OSI Model

- The seven-layer framework can seem more detailed than necessary for some practical tasks.

- Most Internet protocol implementations do not follow all seven layers as separate components.

- The model describes functions rather than prescribing a single practical protocol suite.

Advantages of the TCP/IP Model

- Widely used to describe and implement Internet communication.

- Works across many types and sizes of networks.

- Supports a broad range of applications and link technologies.

- Based on protocols that are implemented in real systems.

Limitations of the TCP/IP Model

- Some functions are not separated as explicitly as they are in OSI.

- It has no distinct Session or Presentation layer in its usual four-layer form.

- Its layer boundaries can be less intuitive when mapping concepts from the OSI model. [1][2]

11. Common Protocols Used in OSI and TCP/IP Models

Protocols are agreed rules that let networked systems exchange data. Different protocols serve different purposes, including naming devices, transferring web content, delivering messages, and routing packets. Together, these protocols allow applications and network hardware to work across different systems. [2][4]

| Protocol | Layer | Purpose |
| --- | --- | --- |
| HTTP | Application | Transfers web requests and responses |
| HTTPS | Application | Protects HTTP traffic using TLS |
| FTP | Application | Transfers files between systems |
| SMTP | Application | Sends email between mail systems |
| POP3 / IMAP | Application | Retrieves or synchronizes email |
| DNS | Application | Looks up domain names |
| TCP | Transport | Reliable, ordered data delivery |
| UDP | Transport | Connectionless datagram delivery |
| IP | Internet | Logical addressing and packet forwarding |
| ICMP | Internet | Error reporting and network diagnostics |
| ARP | Network Access (IPv4 local link) | Finds a local-link address for an IPv4 address |
| Ethernet | Network Access | Wired local network communication |
| Wi-Fi | Network Access | Wireless local network communication |

Brief Description of Important Protocols

HTTP (Hypertext Transfer Protocol):

HTTP defines how a browser and web server exchange requests and responses for web resources.

HTTPS (Hypertext Transfer Protocol Secure):

HTTPS protects HTTP communication by using TLS to encrypt data in transit and help authenticate the server, reducing the risk of others reading or altering the exchange.

FTP (File Transfer Protocol):

FTP is a protocol for transferring files between systems. Other secure transfer methods are often preferred when confidentiality is required.

SMTP (Simple Mail Transfer Protocol):

SMTP is used to send email between clients and mail servers or between mail servers.

DNS (Domain Name System):

DNS looks up domain names and returns information such as the IP address needed to contact a service.www.google.com

TCP (Transmission Control Protocol):

TCP establishes a connection and provides ordered delivery. It detects missing data and retransmits it when necessary.

UDP (User Datagram Protocol):

UDP sends datagrams without setting up a connection or guaranteeing delivery. It can be useful for real-time or delay-sensitive applications when the application can tolerate or manage packet loss.

IP (Internet Protocol):

IP provides logical addressing and forwards packets toward their destination across interconnected networks.

12. Real-World Implementation of OSI and TCP/IP Models

In practical networks, the OSI model is mainly a guide for understanding and troubleshooting, while TCP/IP protocols carry the actual traffic. The examples below show how several layers cooperate during common online activities. [1][2]

12.1 Web Browsing

When someone enters a website address in a browser, the following steps may occur:

- DNS looks up the domain name to find the required network address.

- The browser uses HTTP or HTTPS to request the page or other resources.

- A transport protocol carries the application data; TCP is commonly used for reliable web transfers, while HTTP/3 uses QUIC over UDP.

- IP forwards packets between the device, routers, and the destination network.

- Wi-Fi or Ethernet carries frames across the local connection.

- The web server processes the request and returns the requested content.

- The browser interprets the response and displays the page.

These steps happen quickly because multiple protocols cooperate, each handling a different part of communication. [2][4]

12.2 Email Communication

When an email is sent:

- SMTP transfers outgoing email toward the recipient's mail server.

- A transport protocol, commonly TCP, carries the message between systems.

- IP routes packets toward the receiving mail server.

- The recipient typically accesses stored mail through IMAP or POP3.

The protocols work together to move the message and make it available to the recipient.

12.3 File Transfer

When a user downloads a file from a website:

- HTTPS or another file-transfer protocol carries the file data.

- When TCP is used, it tracks delivery and restores the correct order of received data.

- Missing TCP data can be retransmitted so the transfer can continue correctly.

- The receiving application writes the completed file to local storage.

12.4 Video Streaming

Streaming services may use different transport approaches depending on the application and protocol. UDP-based approaches, including QUIC, can reduce delays, while buffering and retransmission strategies help manage network variation.

If data arrives late or is lost, the player may adjust quality, buffer briefly, or recover through the streaming protocol; the exact effect depends on the service and connection.

12.5 Cloud Computing

Cloud storage platforms use Internet protocols to upload, retrieve, and synchronize files between user devices and remote servers. Authentication and encryption are also important parts of these exchanges.

12.6 Internet of Things (IoT)

Connected devices such as cameras, sensors, smart lights, and wearable devices can use IP-based networks to report measurements, receive commands, and communicate with local hubs or cloud services.

13. Case Study

Case Study: Opening an Online Shopping Website

Background

A customer uses a laptop connected through Wi-Fi to visit an online store and view a smartphone listing. A simple click starts a series of exchanges involving name resolution, secure web requests, transport, routing, and the local network connection.

Communication Process

Step 1:

The customer enters the online store's address in the browser.

Step 2:

DNS resolves the store's domain name to the address needed for connection.

Step 3:

The browser establishes secure web communication and sends an HTTPS request.

Step 4:

The transport protocol carries the request data; for a typical HTTPS connection this may be TCP, while newer HTTP/3 connections use QUIC over UDP.

Step 5:

IP packets are forwarded through routers toward the store's network.

Step 6:

The laptop sends frames across the local Wi-Fi network.

Step 7:

The web server processes the request and retrieves the product information needed to build the page.

Step 8:

The response returns to the laptop, and the browser renders the product page for the customer.

Role of the OSI Model

The OSI model helps break the process into understandable responsibilities:

- The Application layer represents the web service and its requests.

- Presentation-related functions include representing data and protecting it through encryption mechanisms.

- Session-related functions involve managing ongoing exchanges between endpoints or applications.

- The Transport layer carries application data and may provide reliable, ordered delivery.

- The Network layer handles logical addressing and routing.

- The Data Link layer prepares frames for the local network.

- The Physical layer transmits bits over radio, copper, or fibre media.

Role of the TCP/IP Model

The TCP/IP model describes the protocol layers used in the actual exchange:

- The Application layer uses DNS and web protocols such as HTTP/HTTPS.

- The Transport layer may use TCP or UDP with QUIC, depending on the web protocol.

- The Internet layer uses IP to address and forward packets.

- The Network Access layer sends data through Wi-Fi or Ethernet.

Benefits Observed

- The request and response are exchanged through defined network protocols.

- HTTPS helps protect data while it travels between browser and server.

- IP enables packets to cross multiple networks.

- Layering makes the communication process easier to analyse and troubleshoot.

- The same general architecture can support many users and different client devices.

This example demonstrates that OSI is useful for explaining networking functions, while TCP/IP protocols provide the mechanisms used in real Internet communication. [1][2][4]

Reference List

[1] IBM, “What is the OSI model?” IBM Think. Available: https://www.ibm.com/think/topics/osi-modelhttps://www.geeksforgeeks.org/computer-networks/difference-between-osi-model-and-tcp-ip-model/

[2] R. Braden, ed., “Requirements for Internet Hosts — Communication Layers,” RFC 1122, Internet Engineering Task Force, Oct. 1989. Available: https://datatracker.ietf.org/doc/html/rfc1122

[3] Cisco Systems, Inc., “Industrial Ethernet: A Control Engineer’s Guide,” white paper, section “The Open Systems Interconnection Reference Model.” Available: https://www.cisco.com/c/dam/en/us/products/collateral/switches/catalyst-2950-series-switches/prod_white_paper0900aecd8013313e.pdf

[4] IBM, “TCP/IP protocols,” IBM Documentation. Available: https://www.ibm.com/docs/en/cics-ts/5.5.0?topic=concepts-tcpip-protocols

https://www.ijecs.in/index.php/ijecs/article/view/4880

https://www.studocu.com/in/document/savitribai-phule-pune-university/computer-networks/case-study-osi-vs-tcpip-models-comparison-cnl/168289327